# Reading the AD8232 using the ADS1115 via ESPIDF

## AI Overview

To read an ECG sensor (like the AD8232) using an ADS1115 16-bit ADC and an ESP32 via ESP-IDF, initialize the I2C master driver, configure the ADS1115 conversion register and gain via I2C write/read transactions, and poll or trigger continuous data conversions at your target sample rate (e.g., 250Hz–500Hz). [1, 2, 3]

## Hardware Wiring

- VDD: Connect to 3.3V or 5V (matching your analog front end).

- GND: Connect to system ground.

- SCL: Connect to the ESP32 I2C clock pin (e.g., GPIO 22).

- SDA: Connect to the ESP32 I2C data pin (e.g., GPIO 21).

- ADDR: Connect to GND to set the default I2C address (0x48).

- AIN0: Connect to the analog output (OUT) of your ECG signal generator/sensor module.

## ESP-IDF Implementation Steps

- I2C Configuration: Set up i2c_config_t with mode master, correct GPIO pin numbers, enable pull-ups, and set a clock speed of 400kHz. Install the driver using i2c_param_config and i2c_driver_install. [1]

- ADS1115 Setup: Write to the Config Register (Pointer Register = 0x01) to select channel AIN0 (MUX = 100), set the programmable gain amplifier (PGA) range (e.g., ±4.096V), and select continuous conversion mode (MODE = 0). [1, 2, 3]

- Data Read Loop: Read from the Conversion Register (Pointer Register = 0x00) using a 2-byte I2C read operation at regular timed intervals (using a FreeRTOS task delayed by vTaskDelay or driven by a hardware timer for precise ECG sampling).

- Signal Processing: Convert the raw 16-bit two's complement value into voltage steps to analyze heart beats or pass the waveform stream via Wi-Fi/Bluetooth. [1, 2]