# Generate ads1115 API calls using ESP-IDF

NOTE: I have been deeply involved in developing an application that uses the ADS1115 4 channel 16-bit ADC chip, and as a result, some sort of driver is needed to interface with the ESP32, and under Espressif’s ESP-IDF. From what I could tell, there are several third-party API or component libraries that are available for the developer to use, but none that are native to the ESP-IDF package. Each third party has their own particular way of handling an interface with the ESP32, but none are consistent across the spectrum. I have tried to find the best one, but it can be challenging to get it to work with the application currently under development. I have had some not particularly good experiences with drivers for other types of boards – some were incomplete or not tested thoroughly.

Fortunately, I have found that Google AI could generate some ADS1115 API calls, which is in the first section. I have also created a second section, with the results from DeepSeek (general chat, I believe is model “r1”) and the result are there.

# Section 1: Results from Google AI

You can generate raw register-level read and write functions for the ADS1115 utilizing the new master Inter-Integrated Circuit (I2C) driver introduced in ESP-IDF v5.2. [1, 2]

## ADS1115 API Implementation

c

#include "driver/i2c_master.h" #include "esp_err.h" #define ADS1115_REG_POINTER_CONV 0x00 #define ADS1115_REG_POINTER_CONFIG 0x01 // Write 16-bit configuration or threshold register esp_err_t ads1115_write_register(i2c_master_dev_handle_t dev_handle, uint8_t reg_addr, uint16_t value) { // ADS1115 expects: [Pointer Byte, MSB Byte, LSB Byte] uint8_t write_data[3]; write_data[0] = reg_addr; write_data[1] = (uint8_t)(value >> 8); // MSB write_data[2] = (uint8_t)(value & 0xFF); // LSB return i2c_master_transmit(dev_handle, write_data, sizeof(write_data), -1); } // Read 16-bit conversion or configuration register esp_err_t ads1115_read_register(i2c_master_dev_handle_t dev_handle, uint8_t reg_addr, uint16_t *out_value) { uint8_t read_buffer[2] = {0}; // Transmit target register pointer, then read 2 bytes back esp_err_t ret = i2c_master_transmit_receive(dev_handle, &reg_addr, 1, read_buffer, 2, -1); if (ret == ESP_OK) { *out_value = ((uint16_t)read_buffer[0] << 8) ¦ read_buffer[1]; } return ret; } // Convenience function to read the latest conversion result esp_err_t ads1115_read_conversion(i2c_master_dev_handle_t dev_handle, int16_t *raw_result) { return ads1115_read_register(dev_handle, ADS1115_REG_POINTER_CONV, (uint16_t *)raw_result); }

Use code with caution.

## Bus and Device Initialization Context

c

// Example initialization using ESP-IDF 5.2 new driver model i2c_master_bus_handle_t bus_handle; i2c_master_bus_config_t bus_config = { .i2c_port = I2C_NUM_0, .sda_io_num = 21, .scl_io_num = 22, .clk_source = I2C_CLK_SRC_DEFAULT, .glitch_ignore_cnt = 7, .flags.enable_internal_pullup = true, }; i2c_new_master_bus(&bus_config, &bus_handle); i2c_master_dev_handle_t ads_dev_handle; i2c_device_config_t dev_config = { .dev_addr_length = I2C_ADDR_BIT_LEN_7, .device_address = 0x48, // Default ADDR pin connected to GND .scl_speed_hz = 100000, // 100kHz }; i2c_master_bus_add_device(bus_handle, &dev_config, &ads_dev_handle);

If you want, I can provide:

- Full single-shot channel configuration logic with bitmasks for the Programmable Gain Amplifier (PGA) and Multiplexer (MUX)

- Integration using an FreeRTOS task loop for continuous conversions

## Please do add Full single-shot channel configuration logic with bitmasks for the Programmable Gain Amplifier (PGA) and Multiplexer (MUX)

Here is the implementation for single-shot reads with complete Mux and PGA register bitmasks, using the ESP-IDF v5.2 I2C master driver.

## Configuration Bitmasks and Enumerations

c

#include "driver/i2c_master.h" #include "esp_err.h" #include "freertos/FreeRTOS.h" #include "freertos/task.h" // Register Pointers #define ADS1115_REG_POINTER_CONV 0x00 #define ADS1115_REG_POINTER_CONFIG 0x01 // Config Register Bits #define ADS1115_OS_START (1 << 15) // Start single conversion #define ADS1115_OS_STATUS_BUSY (0 << 15) // Read: Conversion in progress #define ADS1115_OS_STATUS_READY (1 << 15) // Read: Conversion complete #define ADS1115_MODE_SINGLE (1 << 8) // Power-down single-shot mode // Multiplexer (MUX) Settings (Bits 14-12) typedef enum { ADS1115_MUX_AIN0_AIN1 = (0x0 << 12), // Differential: AIN0 (+) - AIN1 (-) ADS1115_MUX_AIN0_AIN3 = (0x1 << 12), // Differential: AIN0 (+) - AIN3 (-) ADS1115_MUX_AIN1_AIN3 = (0x2 << 12), // Differential: AIN1 (+) - AIN3 (-) ADS1115_MUX_AIN2_AIN3 = (0x3 << 12), // Differential: AIN2 (+) - AIN3 (-) ADS1115_MUX_AIN0_GND = (0x4 << 12), // Single-ended: AIN0 ADS1115_MUX_AIN1_GND = (0x5 << 12), // Single-ended: AIN1 ADS1115_MUX_AIN2_GND = (0x6 << 12), // Single-ended: AIN2 ADS1115_MUX_AIN3_GND = (0x7 << 12), // Single-ended: AIN3 } ads1115_mux_t; // Programmable Gain Amplifier (PGA) Settings (Bits 11-9) typedef enum { ADS1115_PGA_6_144V = (0x0 << 9), // +/-6.144V range ADS1115_PGA_4_096V = (0x1 << 9), // +/-4.096V range ADS1115_PGA_2_048V = (0x2 << 9), // +/-2.048V range (Default) ADS1115_PGA_1_024V = (0x3 << 9), // +/-1.024V range ADS1115_PGA_0_512V = (0x4 << 9), // +/-0.512V range ADS1115_PGA_0_256V = (0x5 << 9), // +/-0.256V range } ads1115_pga_t; // Data Rate Settings (Bits 7-5) typedef enum { ADS1115_DR_8SPS = (0x0 << 5), ADS1115_DR_16SPS = (0x1 << 5), ADS1115_DR_32SPS = (0x2 << 5), ADS1115_DR_64SPS = (0x3 << 5), ADS1115_DR_128SPS = (0x4 << 5), // Default ADS1115_DR_250SPS = (0x5 << 5), ADS1115_DR_475SPS = (0x6 << 5), ADS1115_DR_860SPS = (0x7 << 5), } ads1115_dr_t; // Comparator Queue Settings (Bits 1-0) - Disables comparator by default #define ADS1115_COMP_QUEUE_DISABLE (0x3 << 0) Use code with caution.

Core API Functions

c

// Lower-level I2C writes using new driver API esp_err_t ads1115_write_register(i2c_master_dev_handle_t dev_handle, uint8_t reg_addr, uint16_t value) { uint8_t write_data[3] = { reg_addr, (uint8_t)(value >> 8), (uint8_t)(value & 0xFF) }; return i2c_master_transmit(dev_handle, write_data, sizeof(write_data), -1); } // Lower-level I2C reads using new driver API esp_err_t ads1115_read_register(i2c_master_dev_handle_t dev_handle, uint8_t reg_addr, uint16_t *out_value) { uint8_t read_buffer[2] = {0}; esp_err_t ret = i2c_master_transmit_receive(dev_handle, &reg_addr, 1, read_buffer, 2, -1); if (ret == ESP_OK) { *out_value = ((uint16_t)read_buffer[0] << 8) ¦ read_buffer[1]; } return ret; } // Single-shot acquisition function esp_err_t ads1115_get_single_shot(i2c_master_dev_handle_t dev_handle, ads1115_mux_t channel, ads1115_pga_t gain, int16_t *raw_result) { // 1. Build config register value uint16_t config = ADS1115_OS_START ¦ channel ¦ gain ¦ ADS1115_MODE_SINGLE ¦ ADS1115_DR_128SPS ¦ ADS1115_COMP_QUEUE_DISABLE; // 2. Write configuration to device esp_err_t ret = ads1115_write_register(dev_handle, ADS1115_REG_POINTER_CONFIG, config); if (ret != ESP_OK) return ret; // 3. Poll Operational Status (OS) bit until conversion completes uint16_t status = 0; int timeout_counter = 50; // Safety exit counter do { vTaskDelay(pdMS_TO_TICKS(5)); // Give the hardware time to sample ret = ads1115_read_register(dev_handle, ADS1115_REG_POINTER_CONFIG, &status); if (ret != ESP_OK) return ret; timeout_counter--; } while (((status & ADS1115_OS_START) == ADS1115_OS_STATUS_BUSY) && timeout_counter > 0); if (timeout_counter <= 0) { return ESP_ERR_TIMEOUT; } // 4. Read data from conversion register return ads1115_read_register(dev_handle, ADS1115_REG_POINTER_CONV, (uint16_t *)raw_result); } Use code with caution.

Application Execution Example

c

void app_main(void) { // Note: Assume i2c_master_dev_handle_t ads_dev_handle is initialized as shown in previous snippet. i2c_master_dev_handle_t ads_dev_handle = NULL; int16_t raw_value = 0; // Example: Read single-ended channel AIN0 with a +/-4.096V range esp_err_t err = ads1115_get_single_shot(ads_dev_handle, ADS1115_MUX_AIN0_GND, ADS1115_PGA_4_096V, &raw_value); if (err == ESP_OK) { // Convert raw values to millivolts (4.096V range -> 4096mV / 32768 counts) float millivolts = raw_value * (4096.0f / 32768.0f); printf("Raw: %d ¦ Voltage: %.2f mV\n", raw_value, millivolts); } else { printf("ADC Read Failed: %s\n", esp_err_to_name(err)); } } Use code with caution.

If you want, I can help you with:

- Setting up the Alert/Ready hardware interrupt pin to remove the polling loop

- Writing a conversion helper for negative differential voltage handling

Let me know what you want to configure next.

# Section 2: Talking with DeepSeek

## I need to find the basic interface code for working with the ESP32, using the ESP-IDF version 5.2.7, which has the more recent driver/i2c_master.h calls. Can you generate the basic read and write I2C register calls?

# ESP-IDF 5.2.7 I2C Master Register Read/Write Code

Based on the ESP-IDF documentation, here's the basic interface code for I2C register operations using the newer driver/i2c_master.h API .

### Core Read/Write Functions

The key functions for register access are i2c_master_transmit_receive() for reading registers (write address, then read data) and i2c_master_transmit() for writing to registers .

c

#include "driver/i2c_master.h" // Write a single byte to a register esp_err_t i2c_register_write_byte(i2c_master_dev_handle_t dev_handle, uint8_t reg_addr, uint8_t data) { uint8_t write_buf[2] = {reg_addr, data}; return i2c_master_transmit(dev_handle, write_buf, sizeof(write_buf), pdMS_TO_TICKS(1000)); } // Read bytes from a register esp_err_t i2c_register_read(i2c_master_dev_handle_t dev_handle, uint8_t reg_addr, uint8_t *data, size_t len) { return i2c_master_transmit_receive(dev_handle, &reg_addr, 1, data, len, pdMS_TO_TICKS(1000)); }

## Full Example (MPU9250 Sensor)

This demonstrates the complete initialization and register read sequence :

c

#include "driver/i2c_master.h" #define I2C_MASTER_SCL_IO GPIO_NUM_22 #define I2C_MASTER_SDA_IO GPIO_NUM_21 #define I2C_MASTER_FREQ_HZ 400000 #define MPU9250_SENSOR_ADDR 0x68 #define MPU9250_WHO_AM_I_REG_ADDR 0x75 void app_main(void) { // 1. Configure and initialize I2C bus i2c_master_bus_handle_t bus_handle; i2c_master_bus_config_t bus_config = { .i2c_port = I2C_NUM_0, .sda_io_num = I2C_MASTER_SDA_IO, .scl_io_num = I2C_MASTER_SCL_IO, .clk_source = I2C_CLK_SRC_DEFAULT, .glitch_ignore_cnt = 7, .flags.enable_internal_pullup = true, }; ESP_ERROR_CHECK(i2c_new_master_bus(&bus_config, &bus_handle)); // 2. Add device to the bus i2c_master_dev_handle_t dev_handle; i2c_device_config_t dev_config = { .dev_addr_length = I2C_ADDR_BIT_LEN_7, .device_address = MPU9250_SENSOR_ADDR, .scl_speed_hz = I2C_MASTER_FREQ_HZ, }; ESP_ERROR_CHECK(i2c_master_bus_add_device(bus_handle, &dev_config, &dev_handle)); // 3. Read WHO_AM_I register to verify connection uint8_t who_am_i = 0; esp_err_t ret = i2c_master_transmit_receive(dev_handle, &MPU9250_WHO_AM_I_REG_ADDR, 1, &who_am_i, 1, pdMS_TO_TICKS(1000)); if (ret == ESP_OK && who_am_i == 0x71) { printf("MPU9250 detected successfully!\n"); } }

## Key Points

- Initialization: Create a bus handle with i2c_new_master_bus(), then add devices with i2c_master_bus_add_device()

- Register read: Use i2c_master_transmit_receive() - sends the register address (write phase), then reads data (read phase)

- Register write: Use i2c_master_transmit() with a buffer containing register address + data

- Timeout: pdMS_TO_TICKS(1000) converts 1000ms to FreeRTOS ticks; -1 means wait forever

- Address: Use 7-bit slave addresses (not including the R/W bit)

## For 16-bit Registers

If your device uses 16-bit register addresses, adjust the write buffer accordingly:

c

esp_err_t i2c_register_write_16(i2c_master_dev_handle_t dev_handle, uint16_t reg_addr, uint8_t data) { uint8_t write_buf[3] = {(reg_addr >> 8) & 0xFF, reg_addr & 0xFF, data}; return i2c_master_transmit(dev_handle, write_buf, sizeof(write_buf), pdMS_TO_TICKS(1000)); }

## Can you do the above for the ADS1115?

The approach for the ADS1115 is the same as the general code, but you'll write to its specific configuration and conversion registers. The modern i2c_master API is the correct way to do this in ESP-IDF 5.2.7 .

### 🧩 The ADS1115 Register Model

To work with the ADS1115, you first write a value to its Pointer Register to select which internal register you want to interact with next. The key registers are:

| Register Name | Pointer Value | Description |
| --- | --- | --- |
| Conversion | 0x00 | Holds the result of the last ADC conversion. |
| Config | 0x01 | Sets the device's operating mode, input channel, gain, and data rate. |
| Lo_Thresh | 0x02 | Lower threshold for the comparator function. |
| Hi_Thresh | 0x03 | Upper threshold for the comparator function. |

You must write to the Config register (Pointer 0x01) to set up the ADS1115, wait for a conversion, and then read the result from the Conversion register (Pointer 0x00) .

### 🚀 Core I2C Operations

Here are the basic functions to write to and read from the ADS1115's registers. They build on the general i2c_master_transmit and i2c_master_transmit_receive functions.

c

#include "driver/i2c_master.h"

// Write a 16-bit value to a specific ADS1115 register

esp_err_t ads1115_write_register(i2c_master_dev_handle_t dev_handle,

uint8_t reg_pointer, uint16_t value) {

// The ADS1115 expects the register address, followed by the 16-bit value (MSB first)

uint8_t write_buf[3] = {

reg_pointer,

(value >> 8) & 0xFF, // High byte

value & 0xFF         // Low byte

};

return i2c_master_transmit(dev_handle, write_buf, sizeof(write_buf), pdMS_TO_TICKS(1000));

}

// Read a 16-bit value from a specific ADS1115 register

esp_err_t ads1115_read_register(i2c_master_dev_handle_t dev_handle,

uint8_t reg_pointer, uint16_t *value) {

uint8_t read_buf[2] = {0};

// First, select which register to read from by sending its pointer value

esp_err_t ret = i2c_master_transmit_receive(dev_handle, &reg_pointer, 1,

read_buf, 2, pdMS_TO_TICKS(1000));

if (ret == ESP_OK) {

// Combine the two bytes into a 16-bit value (MSB first)

*value = ((uint16_t)read_buf[0] << 8) | read_buf[1];

}

return ret;

}

### ⚙️ Putting It Together: Reading a Single Voltage

To read a voltage from channel AIN0 relative to GND, you would set the config register, wait, and then read the conversion result. This example sets the ADS1115 to read at 128 SPS with a ±4.096V range .

c

// I2C device address. ADDR pin tied to GND = 0x48

#define ADS1115_I2C_ADDR    0x48

// Pointer Register values

#define ADS1115_REG_POINTER_CONVERSION  0x00

#define ADS1115_REG_POINTER_CONFIG      0x01

// Config Register bits for:

// AIN0 vs GND, continuous conversion, 128 SPS, ±4.096V

#define ADS1115_CONFIG_VALUE            0x8483 // AIN0 to GND, 128 SPS, FSR ±4.096V

void read_ads1115_ain0(i2c_master_dev_handle_t dev_handle) {

// 1. Write to the Config Register to start a conversion

esp_err_t ret = ads1115_write_register(dev_handle, ADS1115_REG_POINTER_CONFIG, ADS1115_CONFIG_VALUE);

if (ret != ESP_OK) {

ESP_LOGE("ADS1115", "Failed to write config register");

return;

}

// 2. Wait for the conversion to complete (e.g., 10ms for 128 SPS)

vTaskDelay(pdMS_TO_TICKS(10));

// 3. Read the result from the Conversion Register

uint16_t raw_value = 0;

ret = ads1115_read_register(dev_handle, ADS1115_REG_POINTER_CONVERSION, &raw_value);

if (ret != ESP_OK) {

ESP_LOGE("ADS1115", "Failed to read conversion register");

return;

}

// 4. Interpret the result (raw_value is a signed 16-bit integer)

int16_t signed_value = (int16_t)raw_value;

float voltage = 4.096f * ((float)signed_value / 32768.0f);

ESP_LOGI("ADS1115", "Raw: 0x%04X, Voltage: %.3f V", raw_value, voltage);

}

### 🧠 Key Points for the ADS1115

- Initialization: The I2C bus and device handles are initialized exactly as shown in the general code example.

- Data Format: The ADS1115 expects and returns 16-bit values in big-endian format (most significant byte first), as shown in the code above .

- Conversion Time: Always wait a sufficient time for the ADC to complete a conversion after configuring it . The required time depends on the chosen data rate. For 128 SPS, ~10ms is safe.

- Pre-built Drivers: For a more robust solution, you can use pre-built ESP-IDF components like bammer187/ads1115 or espp/ads1x15, which handle the full register configuration and can be added via idf.py add-dependency .

Let me know if you need help adapting this for a specific measurement, like differential inputs or a particular sample rate!