# Using DeepSeek to assist in the Design of the Filter Cascade in the Pan Tompkins QRS algorithm

# What are the digital filter types used in the Pan Tompkins QRS detector algorithm?

Read 11 web pages

The Pan-Tompkins QRS detection algorithm uses a specific cascade of digital filters to isolate the QRS complex from noise and other ECG waves . The primary digital filter types used are a band-pass filter, a derivative filter, and a moving-window integrator (a type of low-pass filter) .

Here's a breakdown of each filter and its purpose:

| Filter Type | Purpose |
| --- | --- |
| Band-pass Filter | Reduces noise like muscle artifacts, power line interference, and baseline wander. It is often implemented as a cascade of a low-pass and a high-pass filter . |
| Derivative Filter | Provides a large response at the high slopes of the QRS complex, distinguishing it from the slower P and T waves . |
| Moving-Window Integrator | Acts as a low-pass filter to smooth the signal and generate a single, broad peak for each QRS complex, making detection easier . |

### ⚙️ Detailed Filter Specifications

While implementations can vary based on the sampling frequency, the original algorithm was designed for a 200 Hz sampling rate. The filter characteristics for this rate are well-documented .

- Band-pass Filter (5–15 Hz)

  - Low-Pass Filter (Cutoff ~11 Hz): The transfer function used is:
H(z) = ((1 - z⁻⁶)² / (1 - z⁻¹)²) .
Its difference equation is:
y(nT) = 2y(nT-T) - y(nT-2T) + x(nT) - 2x(nT-6T) + x(nT-12T) .

  - High-Pass Filter (Cutoff ~5 Hz): The transfer function is:
H(z) = ((-1/32 + z⁻¹⁶ - z⁻¹⁷ + z⁻³²/32) / (1 - z⁻¹)) .
The implementation equation from a reliable source is :
y(nT) = 32x(nT - 16T) - y(nT - T) - x(nT) + x(nT - 32T)

- Derivative Filter : A 5-point derivative filter is used, often with a transfer function designed to be causal for real-time use:
H(z) = (1/8) * (2 + z⁻¹ - z⁻³ - 2z⁻⁴) .
This corresponds to a difference equation where the output is based on the current and previous input samples .

- Moving-Window Integrator : This is a moving-average filter that sums the previous N samples. The window length N is set to approximately 160 ms to encompass the entire QRS complex, which is 32 samples at 200 Hz .

Modern implementations may use different filters (like Butterworth) to achieve the same band-pass effect , but the functional stages remain true to the original design.

# Espressif has a DSP library called esp-dsp. Which modules would I want to use in order to create the filters described above for the Pan Tompkins QRS detector?

Read 15 web pages

To implement the Pan-Tompkins QRS detector on an ESP32 using the esp-dsp library, you would primarily use the IIR (Biquad) and FIR filter modules, combined with basic vector math functions for the squaring and windowing operations. While esp-dsp doesn't have a dedicated "moving-window integrator" function, this can be efficiently implemented using a standard moving-average approach.

Here is a breakdown of which esp-dsp modules correspond to each stage of the Pan-Tompkins algorithm.

| Algorithm Stage | esp-dsp Module / Function | Implementation Notes |
| --- | --- | --- |
| Band-pass Filter (Low-pass + High-pass) | IIR (dsps_biquad_f32) or FIR (dsps_fir_f32) | Use a cascade of biquad filters (dsps_biquad_gen_lpf_f32 and dsps_biquad_gen_hpf_f32) to create the 5-15 Hz band-pass . |
| Derivative Filter | FIR (dsps_fir_f32) | Implement the 5-point derivative filter as an FIR filter with specific coefficients . |
| Squaring | Basic Math | No specific esp-dsp module is strictly necessary for a simple element-wise squaring operation. |
| Moving-Window Integrator | Custom Implementation | The integrator is a moving average filter. This can be implemented from scratch (e.g., using a circular buffer and maintaining a running sum) or by using a generic FIR filter with all coefficients equal to 1/N . |

### 🔧 A Practical Implementation Guide

Based on how others have used esp-dsp for similar audio and signal processing tasks, here are the practical steps for building your filter pipeline.

- Building the Band-Pass Filter: The original Pan-Tompkins algorithm uses a cascade of a low-pass and high-pass filter to form the band-pass . esp-dsp provides convenient functions to generate the coefficients for standard biquad filters. The dsps_biquad_gen_lpf_f32 function can create your low-pass filter (cutoff ~11 Hz), and dsps_biquad_gen_hpf_f32 can create your high-pass filter (cutoff ~5 Hz) . You would then cascade these filters by passing the output of one into the input of the other using dsps_biquad_f32 .

- Derivative and Moving-Average Stages: After the band-pass stage, you can process the signal block by block.

  - Derivative: The derivative filter is best implemented as a FIR filter (dsps_fir_f32) using its specific coefficients .

  - Squaring: This is a simple element-wise operation: output[i] = input[i] * input[i].

  - Moving-Window Integrator: This creates a moving average over a window of ~150ms. The most efficient way to implement this on a microcontroller is typically not to call a library function on every sample, but to use a custom algorithm with a circular buffer and a running sum, which the standard esp-dsp functions don't directly provide. Alternatively, you can model it as an FIR filter with all coefficients set to 1/N, where N is the window size .

### 🧮 Optimizing the Filter Chain

The esp-dsp library is designed for efficiency, especially on ESP32 chips with vector instructions.

- Batch Processing: All the dsps_* functions process arrays of data in a single call (e.g., dsps_biquad_f32 processes a buffer of N samples at once) . Your application should collect a block of ECG samples (e.g., 128 or 256 samples) and then pass the entire block through each stage of the pipeline. This is much more efficient than filtering sample-by-sample.

- Data Types: The esp-dsp documentation highlights benchmarks for both f32 (floating-point) and s16 (fixed-point) functions . For a biomedical application, floating-point is easier to work with and avoids precision issues. An example from the esp32.com forums shows a typical workflow:

  - Read 16-bit integer samples (e.g., from an ADC).

  - Convert the block of samples to a floating-point array.

  - Apply the esp-dsp filter functions to the floating-point array.

  - Convert the filtered block back to integers for further processing or output .

### 💡 A Note on Ecosystem Components

While esp-dsp is the core library, the ESP-IDF ecosystem also offers higher-level components that can simplify your work. For example, the espp/filters component provides a C++ BiquadFilter class that wraps the esp-dsp biquad functions, making the code potentially more straightforward to manage .