# Circuit designs for battery powered ECG monitor so far

I decided to see what Google AI would come up with, given the known components and conditions so far, and this is the result:

# (What is a) circuit for powering esp32s3 super mini board with battery and mt3608 and battery charging with tp4056

To power an ESP32-S3 Super Mini board with a 3.7V Lithium battery using a TP4056 charger and an MT3608 boost converter, wire the TP4056 OUT+/OUT- to the MT3608 input, adjust the MT3608 output precisely to 5V (or 3.3V depending on the pin), and feed it to the board's 5V and GND pins. [1, 2, 3, 4]

## Component Wiring Guide

- TP4056 Module (with Battery Protection)

  - IN+ / IN-: Connect to a 5V USB power source for charging the cell.

  - B+ / B-: Connect directly to the positive and negative terminals of your 3.7V Li-ion/LiPo battery.

  - OUT+ / OUT-: Connect to the input of the boost converter (these terminals supply the protected battery voltage or pass-through USB power). [1, 2, 3, 4, 5]

- MT3608 Boost Converter Module

  - VIN+ / VIN-: Connect to the OUT+ and OUT- of the TP4056.

  - VOUT+ / VOUT-: Adjust the on-board blue potentiometer before connecting the ESP32 until a multimeter reads a stable 5.0V across the output. [1, 2]

- ESP32-S3 Super Mini Board

  - 5V (or VIN) Pin: Connect to the VOUT+ of the MT3608.

  - GND Pin: Connect to the VOUT- of the MT3608 and common ground back to the TP4056. [1, 2]

## Critical Precautions & Operation

- Adjust Voltage First: Never hook the ESP32-S3 Super Mini to the MT3608 before turning the potentiometer screw to dial in 5V, as they often ship with the output wide open at high or unstable voltages. [1]

- Quiescent Current Draw: The MT3608 module has an idle/quiescent current draw that will slowly drain your battery even when the ESP32 is in deep sleep. For ultra-low power projects, place a physical slide switch between the TP4056 OUT+ and the MT3608 VIN+ input. [1]

# The circuit must also use an ADS1115 and DS3231 and MicroSD card interfaced with SDMMC.

To interface the ADS1115 (ADC), DS3231 (RTC), and a MicroSD Card via SDMMC with your ESP32-S3 Super Mini, you will expand the previous power circuit and configure the target communication buses (I2C and SDMMC).

The ESP32-S3 features a dedicated hardware peripheral for SDMMC, but because the Super Mini has limited exposed GPIO pins, pin assignment must be chosen carefully to avoid conflicts with standard strapping pins.

## Complete Schematic & Wiring Layout

### 1. Power Distribution Rails

Keep your 5V power supply line dedicated to the components that require it, and rely on the ESP32-S3's internal voltage regulator to provide 3.3V for the SD card and I2C peripherals.

| From (Source) | To (Destination Component & Pin) | Notes |
| --- | --- | --- |
| MT3608 VOUT+ (5.0V) | ESP32-S3 5V pin, DS3231 VCC pin | Powers ESP32 regulator & RTC |
| MT3608 VOUT- (GND) | ESP32 GND, DS3231 GND, ADS1115 GND, MicroSD GND | Common ground rail |
| ESP32-S3 3V3 pin (3.3V) | ADS1115 VDD, MicroSD Card 3V3 | Safe logic voltage rail |

### 2. I2C Bus Connections (ADS1115 & DS3231)

Both the ADS1115 and DS3231 share the exact same I2C data lines. You can assign any free digital pins on the ESP32-S3 for I2C using software initialization, but GPIO 1 (SDA) and GPIO 2 (SCL) are common choices on this mini form factor.

- Pull-up Resistors: The DS3231 module usually includes onboard 4.7kΩ pull-up resistors to its VCC line. If your DS3231 is powered at 5V, ensure your ESP32-S3 pins are 5V tolerant, or power the DS3231 at 3.3V instead if its internal pull-ups interfere with 3.3V logic. Alternatively, use a level shifter.

- ADS1115 Address: Connect the ADDR pin on the ADS1115 to GND to set its default I2C address to 0x48. [1, 2, 3, 4]

| ESP32-S3 Pin | ADS1115 Pin | DS3231 Pin | Hardware Function |
| --- | --- | --- | --- |
| GPIO 1 | SDA | SDA | I2C Data |
| GPIO 2 | SCL | SCL | I2C Clock |

### 3. SDMMC Bus Connections (MicroSD Card Module)

The ESP32-S3 supports 1-bit or 4-bit SDMMC modes. For a "Super Mini" project where saving pins is critical, 1-bit SDMMC mode is recommended because it only requires 3 data/clock lines instead of 6, while still providing significantly faster speeds than standard SPI.

Using the primary hardware SDMMC Slot 1 peripheral, wire the MicroSD card slot directly to these specific ESP32-S3 hardware pins:

| ESP32-S3 Pin | MicroSD Card Pin | SDMMC Function (1-bit Mode) | Required Pull-up |
| --- | --- | --- | --- |
| GPIO 12 | CLK | Clock | None |
| GPIO 11 | CMD | Command Line | External 10kΩ to 3.3V |
| GPIO 13 | D0 (DAT0) | Data Line 0 | External 10kΩ to 3.3V |

Note: If using a generic MicroSD breakout card board, ensure it does not have built-in 5V-to-3.3V level shifters, as you are running the lines directly at 3.3V logic. [1]

### Code Initialization Snippet (Arduino IDE)

When initializing these peripherals in your code, you must explicitly declare the custom I2C pins and call the SDMMC peripheral driver correctly.

cpp

#include <Wire.h> #include <Adafruit_ADS1X15.h> #include <RTClib.h> #include "FS.h" #include "SD_MMC.h" // Define custom I2C pins for ESP32-S3 Super Mini #define I2C_SDA 1 #define I2C_SCL 2 Adafruit_ADS1115 ads; RTC_DS3231 rtc; void setup() { Serial.begin(115200); // 1. Initialize I2C Bus with custom pins Wire.begin(I2C_SDA, I2C_SCL); // 2. Initialize Peripherals if (!ads.begin(0x48)) { Serial.println("ADS1115 initialization failed!"); } if (!rtc.begin()) { Serial.println("DS3231 initialization failed!"); } // 3. Initialize SDMMC in 1-bit mode using standard pins (GPIO 11, 12, 13) // On ESP32-S3, Slot 1 pins are fixed to 11 (CMD), 12 (CLK), and 13 (D0) if(!SD_MMC.begin("/sdcard", true)) { // The 'true' parameter forces 1-bit mode Serial.println("Card Mount Failed"); return; } Serial.println("SDMMC Card initialized successfully."); } void loop() { // Your data logging code here }

Would you like help calculating the expected battery life runtime for this circuit, or do you need assistance configuring the ADS1115 gain settings for specific sensor voltages?

# The ADS1115 will interface with 4 AD8232 heart monitor boards. What is the battery life runtime for this circuit?

Assuming you are using a standard 2500 mAh 18650 Lithium-ion battery, your circuit will run for approximately 18 to 22 hours under continuous data-logging operation, or up to several months if utilizing optimized duty cycles and deep sleep. [1, 2, 3]

Because the component power profiles change drastically based on your software implementation, the calculations below break down the exact battery life runtime for both Continuous Logging and Periodic Sleep scenarios.

## 1. Total System Current Draw Analysis

To calculate runtime, we first map out the active current consumption of every component in the loop.

- ESP32-S3 Super Mini: Draws ~100 mA under active CPU processing (at 240MHz). If you initialize Wi-Fi or Bluetooth, this spikes to a continuous ~120-150 mA. [1, 2]

- 4× AD8232 Heart Monitors: Each AD8232 integrated circuit consumes roughly 170 µA. For 4 boards, this totals ~0.68 mA (4 × 0.17 mA). [1, 2, 3, 4]

- ADS1115 ADC: Consumes a negligible 0.15 mA in continuous conversion mode. [1]

- DS3231 RTC: Consumes about 0.1 mA when actively reading via I2C.

- MicroSD Card via SDMMC: Draws roughly 0.5 mA when idle, but surges to ~80 mA for milliseconds during a write/flush process. Over continuous blocks, its average consumption settles around ~20 mA.

- MT3608 Boost Converter Efficiency: The MT3608 operates at roughly 85% efficiency stepping up from a 3.7V battery to 5.0V. We must multiply the total current draw by ~1.57 to account for the efficiency loss and the voltage conversion ratio (5.0V / 3.7V / 0.85).

## 2. Runtime Scenarios (Based on a 2500 mAh Battery)

### Scenario A: Continuous Monitoring & Logging (Worst-Case)

The ESP32-S3 stays fully awake, reading 4 analog channels sequentially from the ADS1115 and streaming/writing data to the SD card without stopping. [1]

- Total Component Draw (at 5V / 3.3V): \(100\text{ mA} \text{ (ESP32)} + 20\text{ mA} \text{ (SD Card)} + 0.68\text{ mA} \text{ (AD8232s)} + 0.15\text{ mA} \text{ (ADC)} + 0.1\text{ mA} \text{ (RTC)} \approx \mathbf{121\text{ mA}}\)

- Battery-Side Draw (accounting for MT3608 step-up & losses): \(121\text{ mA} \times 1.57 = \mathbf{190\text{ mA}}\)

- Runtime: \(\frac{2500\text{ mAh}}{190\text{ mA}} \approx \mathbf{13.1\text{ hours}}\)

- Note: If you run the ESP32-S3 at a reduced clock speed (e.g., 80 MHz instead of 240 MHz) and disable Wi-Fi/Bluetooth, the ESP32 draw drops to ~40 mA, expanding your runtime to ~22.5 hours. [1]

NOTE: The mathematical formula formatting can be a bit confusing, so an image snapshot of the above shows:

### Scenario B: Duty-Cycled Power Logging (Optimized Case)

The system wakes up once every 10 minutes, samples heart rate data for 30 seconds, saves it to the MicroSD card, and drops back into deep sleep. [1, 2]

- Active State (30 seconds): Draws 190 mA from the battery (same as Scenario A).

- Deep Sleep State (9.5 minutes): The ESP32-S3 turns off its main cores, leaving the RTC alive. Because the MT3608 booster remains active and the AD8232 modules lack automatic power-down switches, they continuously pull quiescent current. Total sleep draw is around ~4 mA.

- Average Current Draw:
\(\frac{(190\text{\ mA}\times 30\text{s})+(4\text{\ mA}\times 570\text{s})}{600\text{s}}=\mathbf{13.3}\text{\ mA\ average}\)

- Runtime: \(\frac{2500\text{ mAh}}{13.3\text{ mA}} \approx 188\text{ hours} \approx \mathbf{7.8\text{ days}}\) [1, 2]

NOTE: Again, the mathematical formula formatting can be a bit confusing, so an image snapshot of the above shows:

## Hardware Tuning Tips to Extend Battery Life

- Implement an External Power Gate: To achieve months of battery life, use a P-channel MOSFET or a low-side N-channel MOSFET connected to an ESP32 GPIO. Use it to cut off power completely to the MT3608 booster and the 4 AD8232 boards during deep sleep. This drops deep sleep consumption down to micro-amps.

- Utilize the AD8232 Shutdown (SDN) Pins: Each AD8232 module has a SDN pin. Tie these pins to a single GPIO on your ESP32-S3. Pulling this line LOW before entering deep sleep drops the current of all four monitor boards to less than 4 µA combined. [1, 2, 3, 4, 5]

If you want to move forward with power optimization, let me know:

- What is your target battery capacity (mAh) or intended physical battery size?

- Can your application support periodic logging (sleeping between reads), or does it require 100% continuous data capture?

I had left the query alone for a day, and when I attempted to add on an additional detail: use the SDMMC in 4-bit mode, the AI  agent (rightfully) responded with “Something went wrong and the content wasn't generated.”

So I restarted the query for that added condition of the SDMMC 4-bit mode interface to the ESP32S3 super mini board:

# Let's start over: i want to a circuit for powering esp32s3 super mini board with battery and mt3608 and battery charging with tp4056, along with 4-bit mode interface via SDMMC to the esp32s3 super mini board. Can you do that?

Yes, you can power an ESP32-S3 Super Mini using a LiPo battery with a TP4056 charger module and an MT3608 boost converter, while using the 4-bit SDMMC mode interface for an SD card. [1, 2]

## Power Circuit Wiring

- Battery to TP4056: Connect the positive terminal of your 3.7V LiPo battery to the B+ pad on the TP4056 module, and the negative terminal to the B- pad.

- TP4056 to MT3608: Connect the OUT+ (or VCC) and OUT- (or GND) pins of the TP4056 to the IN+ and IN- pins of the MT3608 boost converter module. [1, 2]

- Setting the Boost Voltage: Adjust the potentiometer on the MT3608 module before connecting it to your microcontroller. Turn it until the output terminals (OUT+ and OUT-) read a steady 5.0V using a multimeter. [1]

- MT3608 to ESP32-S3: Connect the output positive OUT+ from the MT3608 to the 5V pin on the ESP32-S3 Super Mini**, and connect OUT- to a GND pin on the board. [1, 2]

## 4-Bit SDMMC Mode Wiring

The ESP32-S3 hardware SDMMC host controller can map 4-bit SD/MMC signals to virtually any flexible GPIO pin via its internal GPIO matrix. Pick standard, non-conflicting digital GPIOs on your ESP32-S3 Super Mini (avoiding strapping pins like GPIO0, GPIO45, and GPIO46 if possible during boot). [1, 2, 3, 4]

Connect the Micro SD Card module pins to your chosen ESP32-S3 GPIOs: [1]

- CLK (Clock): Connect to a free GPIO (e.g., GPIO 39)

- CMD (Command / DI): Connect to a free GPIO (e.g., GPIO 40)

- D0 (Data 0): Connect to a free GPIO (e.g., GPIO 41)

- D1 (Data 1): Connect to a free GPIO (e.g., GPIO 42)

- D2 (Data 2): Connect to a free GPIO (e.g., GPIO 1)

- D3 (Data 3 / CS): Connect to a free GPIO (e.g., GPIO 2)

- VCC: Connect to the 3V3 pin of the ESP32-S3 board (Micro SD cards run strictly on 3.3V).

- GND: Connect to system GND.

- Pull-up Resistors: Add external 10kΩ pull-up resistors on CMD, D0, D1, D2, and D3 lines to ensure stable high-speed 4-bit data transfers. [1, 2, 3]

If you want, I can provide:

- The specific C++ code snippet using ESP-IDF or Arduino to initialize the 4-bit SDMMC driver

- A recommended list of safe GPIO pins based on your specific Super Mini pinout variant

# The circuit also requires an I2c connection between the esp32s3 super mini board and a DS3231 RTC, and the ads1115 ADC

To add the DS3231 RTC and ADS1115 ADC to your circuit, you will connect both devices to the same I2C bus on the ESP32-S3. Since I2C is a shared bus protocol, both devices can share the exact same SDA and SCL pins because they have different, unique hardware addresses. [1, 2, 3, 4]

## Power Setup for I2C Devices

- DS3231 RTC Power: Connect its VCC pin to the 3.3V pin of the ESP32-S3. Connect GND to common system ground.

- ADS1115 ADC Power: Connect its VDD pin to the 3.3V pin of the ESP32-S3. Connect GND to common system ground.

- Pull-up Resistors: Both the SDA and SCL lines require a 4.7kΩ to 10kΩ pull-up resistor tied to the 3.3V rail. Note that many off-the-shelf DS3231 and ADS1115 breakout modules already include these built-in pull-up resistors, so you may not need to add external ones. [1, 2, 3]

## I2C Bus Wiring

Choose two available GPIO pins on your ESP32-S3 Super Mini to act as the hardware I2C bus. For example, you can use GPIO 4 for SDA and GPIO 5 for SCL: [1, 2]

- SDA (Serial Data): Connect the SDA pin of the DS3231, the SDA pin of the ADS1115, and GPIO 4 of the ESP32-S3 all together.

- SCL (Serial Clock): Connect the SCL pin of the DS3231, the SCL pin of the ADS1115, and GPIO 5 of the ESP32-S3 all together.

- ADS1115 ADDR Pin: Connect this pin to GND to assign the ADS1115 its standard default I2C address (0x48). [1, 2, 3]

## Updated Pin Allocation Map

To prevent resource conflicts with your 4-bit SDMMC setup, ensure your pins are allocated distinctly:

- I2C Bus: GPIO 4 (SDA), GPIO 5 (SCL)

- SDMMC Bus: GPIO 39 (CLK), GPIO 40 (CMD), GPIO 41 (D0), GPIO 42 (D1), GPIO 1 (D2), GPIO 2 (D3) [1, 2]

If you'd like, I can:

- Provide an Arduino sketch that initializes both the 4-bit SD Card and scans the I2C bus for the RTC and ADC

- Help you calculate the battery life based on your estimated power consumption [1]

# I will be using the vs code esp-idf extension, with the ADC capture at 860 samples per second, with all 4 ADC channels in continuous capture

Here is your file:

main

c

Running the ADS1115 ADC at its maximum 860 samples per second (SPS) across all 4 channels in continuous capture mode introduces specific timing, data management, and firmware architecture requirements. [1]

## Architectural Constraints for 860 SPS

Because the ADS1115 contains only a single internal analog-to-digital converter core, it utilizes an analog multiplexer (MUX) to cycle between input pins. True "simultaneous" continuous capture across 4 unique channels is impossible on a single chip. Instead, you must rapidly cycle the MUX configuration between your four channels.

To handle this efficiently using the native ESP-IDF freeRTOS ecosystem inside VS Code, the generated file implements a split task-handling framework:

- Continuous Switching Bottleneck: At 860 SPS, a single conversion cycle takes exactly 1.16 milliseconds. Every time your code modifies the MUX register to sample a different channel, the ADS1115 invalidates its current cycle and restarts its conversion window. Your firmware must wait at least 1.2 to 2 milliseconds before reading the newly settled data register.

- Dual-Core Task Separation: Writing data directly to an SD Card via SDMMC causes non-deterministic blocking delays while the physical flash memory clears its sectors. If you attempt to sample the I2C bus and write to the file on the exact same execution thread, you will drop a massive percentage of your samples.

- Queue-Buffered Flow: The provided code handles high-speed sample execution by creating a dedicated high-priority acquisition thread locked onto Core 0 which polls the I2C bus. It packs the sequential data points into a struct batch and pipes it through a memory queue over to a lower-priority file-handling thread locked onto Core 1.

## Configuration Register Setup for ESP-IDF

To instruct the ADS1115 to sample at maximum speed, you must pass specific binary configurations to its internal config register (Register 1). The bitfields are structured as follows:

- Bit 15: 1 (Begins conversion / operational status)

- Bits [14:12]: MUX input options (100 for AIN0, 101 for AIN1, 110 for AIN2, 111 for AIN3)

- Bits [11:9]: Programmable Gain Amplifier selection (010 assigns full-scale range to ±2.048V)

- Bit 8: Operating state mode (0 forces continuous conversion mode)

- Bits [7:5]: Data rate selector (111 maps to the hardware limit of 860 SPS)

- Bits [4:0]: Comparator control queues (00011 turns off the physical alert pin functions)

This resolves to the following sequence of execution hex parameters built into the attached main.c script:

c

uint16_t ch_configs[4] = {

0x44E0, // AIN0 vs GND, 860 SPS, Continuous

0x54E0, // AIN1 vs GND, 860 SPS, Continuous

0x64E0, // AIN2 vs GND, 860 SPS, Continuous

0x74E0  // AIN3 vs GND, 860 SPS, Continuous

};

Use code with caution.

If you would like, we can expand on this by:

- Configuring an External Hardware Interrupt Line linking the ADS1115 ALERT/RDY pin to an ESP32-S3 GPIO pin to eliminate the software-timed wait loops entirely.

- Drafting the matching CMakeLists.txt configuration files or sdkconfig parameters required to build this code error-free in your VS Code workspace.

Let me know how you would like to tune your project!

# Please add the external HW interrupt ALERT pin from the ADS1115 to the ESP32S3 super mini board. Also, draft the CmakeLists.txt configuration files needed.

To implement a reliable 860 SPS continuous capture across all 4 channels without wasting CPU cycles in delays, you must connect the hardware ALERT/RDY pin of the ADS1115 to an ESP32-S3 GPIO pin.

When configured correctly, the ADS1115 drops this pin low the exact instant a conversion finishes. This triggers an Edge-Triggered Interrupt Service Routine (ISR) in ESP-IDF, waking up your sensor reading task instantly. [1, 2]

## Hardware Connections & Pull-Up Requirement

- ALERT/RDY Pin Connection: Connect the ALERT pin of the ADS1115 to GPIO 6 on your ESP32-S3 Super Mini board.

- Pull-Up Resistor: The ALERT pin is an open-drain output. You must connect a physical 10kΩ pull-up resistor between the ALERT pin line and the 3.3V rail to allow the line to snap back high when idle.

## ESP-IDF Source Implementation (main.c)

This updated C implementation utilizes a FreeRTOS Binary Semaphore given directly from the hardware GPIO ISR to unblock the acquisition task. Because you need to cycle through all 4 channels sequentially, every time the ISR fires, the code reads the current channel's value, alters the internal ADS1115 MUX register to point to the next channel, and sleeps until the next interrupt.

c

#include <stdio.h> #include "freertos/FreeRTOS.h" #include "freertos/task.h" #include "freertos/queue.h" #include "freertos/semphr.h" #include "driver/i2c.h" #include "driver/gpio.h" #include "esp_vfs_fat.h" #include "sdmmc_cmd.h" #define I2C_MASTER_SDA_IO 4 #define I2C_MASTER_SCL_IO 5 #define I2C_MASTER_NUM I2C_NUM_0 #define I2C_MASTER_FREQ_HZ 400000 #define ADS1115_I2C_ADDR 0x48 #define ADS1115_CONV_REG 0x00 #define ADS1115_CFG_REG 0x01 #define ADS_ALERT_GPIO 6 #define ESP_INTR_FLAG_DEFAULT 0 // Data structures for cross-core buffering typedef struct { uint32_t timestamp; int16_t channels[4]; } adc_sample_t; static QueueHandle_t adc_queue = NULL; static SemaphoreHandle_t ads_rdy_semaphore = NULL; // Channel MUX array configuration for 860 SPS, Continuous Mode, FSR = +/-2.048V // Traditional continuous configuration but with ALERT/RDY pin assertion enabled static const uint16_t ads_mux_config[4] = { 0x44E8, // Channel 0 to GND, 860 SPS, Continuous, Alert Active Low 0x54E8, // Channel 1 to GND, 860 SPS, Continuous, Alert Active Low 0x64E8, // Channel 2 to GND, 860 SPS, Continuous, Alert Active Low 0x74E8 // Channel 3 to GND, 860 SPS, Continuous, Alert Active Low }; // Hardware Interrupt Service Routine static void IRAM_ATTR ads_gpio_isr_handler(void* arg) { BaseType_t xHigherPriorityTaskWoken = pdFALSE; xSemaphoreGiveFromISR(ads_rdy_semaphore, &xHigherPriorityTaskWoken); if (xHigherPriorityTaskWoken) { portYIELD_FROM_ISR(); } } // Inline I2C helper write functions static esp_err_t ads1115_write_reg(uint8_t reg, uint16_t val) { uint8_t out_buf[3] = { reg, (uint8_t)(val >> 8), (uint8_t)(val & 0xFF) }; return i2c_master_write_to_device(I2C_MASTER_NUM, ADS1115_I2C_ADDR, out_buf, 3, pdMS_TO_TICKS(10)); } static esp_err_t ads1115_read_reg(uint8_t reg, uint16_t *val) { uint8_t in_buf[2]; esp_err_t err = i2c_master_write_read_device(I2C_MASTER_NUM, ADS1115_I2C_ADDR, &reg, 1, in_buf, 2, pdMS_TO_TICKS(10)); if (err == ESP_OK) { *val = (in_buf[0] << 8) ¦ in_buf[1]; } return err; } // High Priority Core 0 Data Acquisition Thread void adc_acquisition_task(void *pvParameters) { uint8_t current_channel = 0; adc_sample_t current_batch = {0}; uint16_t raw_reading = 0; // Step 1: Force active continuous sequence conversion on first channel ads1115_write_reg(ADS1115_CFG_REG, ads_mux_config[current_channel]); while (1) { // Wait for ALERT pin to flag active reading completion if (xSemaphoreTake(ads_rdy_semaphore, portMAX_DELAY) == pdTRUE) { // Read valid data output register if (ads1115_read_reg(ADS1115_CONV_REG, &raw_reading) == ESP_OK) { current_batch.channels[current_channel] = (int16_t)raw_reading; } // Immediately switch multiplexer configuration target to prepare the next conversion line uint8_t next_channel = (current_channel + 1) % 4; ads1115_write_reg(ADS1115_CFG_REG, ads_mux_config[next_channel]); // If a complete 4-channel sweeping loop finished, push structured metrics into queue buffer if (current_channel == 3) { current_batch.timestamp = xTaskGetTickCount() * portTICK_PERIOD_MS; xQueueSend(adc_queue, &current_batch, 0); } current_channel = next_channel; } } } // Standard Core 1 Low Priority SD Card Writer Thread void sd_writer_task(void *pvParameters) { adc_sample_t log_data; while (1) { if (xQueueReceive(adc_queue, &log_data, portMAX_DELAY) == pdTRUE) { FILE* f = fopen("/sdcard/adc_log.csv", "a"); if (f != NULL) { fprintf(f, "%lu,%d,%d,%d,%d\n", log_data.timestamp, log_data.channels[0], log_data.channels[1], log_data.channels[2], log_data.channels[3]); fclose(f); } } } } void app_main(void) { // 1. Setup I2C Peripheral i2c_config_t conf = { .mode = I2C_MODE_MASTER, .sda_io_num = I2C_MASTER_SDA_IO, .sda_pullup_en = GPIO_PULLUP_ENABLE, .scl_io_num = I2C_MASTER_SCL_IO, .scl_pullup_en = GPIO_PULLUP_ENABLE, .master.clk_speed = I2C_MASTER_FREQ_HZ, }; i2c_param_config(I2C_MASTER_NUM, &conf); i2c_driver_install(I2C_MASTER_NUM, conf.mode, 0, 0, 0); // 2. Setup ALERT GPIO Interrupt Service Structure gpio_config_t io_conf = { .intr_type = GPIO_INTR_NEGEDGE, // Alert pin assertions force low pull drops .pin_bit_mask = (1ULL << ADS_ALERT_GPIO), .mode = GPIO_MODE_INPUT, .pull_up_en = GPIO_PULLUP_ENABLE // Internal backup safeguard pull-up }; gpio_config(&io_conf); ads_rdy_semaphore = xSemaphoreCreateBinary(); adc_queue = xQueueCreate(50, sizeof(adc_sample_t)); gpio_install_isr_service(ESP_INTR_FLAG_DEFAULT); gpio_isr_handler_add(ADS_ALERT_GPIO, ads_gpio_isr_handler, NULL); // [SDMMC Initializations omitted here for snippet clarity but mapping pins 39,40,41,42,1,2] // 3. Pin tasks to unique execution physical processor cores xTaskCreatePinnedToCore(adc_acquisition_task, "ADC_Acq", 4096, NULL, 10, NULL, 0); xTaskCreatePinnedToCore(sd_writer_task, "SD_Write", 4096, NULL, 4, NULL, 1); }

Use code with caution.

## Workspace Project Build Architecture Configuration

Inside modern VS Code ESP-IDF extension setups, configuration systems depend on two distinct CMakeLists.txt scripts tracking component configurations.

### 1. Root Directory Script (/CMakeLists.txt)

Place this file into the root folder directory of your workspace layout:

cmake

cmake_minimum_required(VERSION 3.16)

include($ENV{IDF_PATH}/tools/cmake/project.cmake)

project(esp32s3_ads1115_sdmmc_datalogger)

### 2. Component Source Directory Script (/main/CMakeLists.txt)

Place this file directly into your inner /main folder containing your code source logic:

cmake

idf_component_register(SRCS "main.c"

INCLUDE_DIRS "."

PRIV_REQUIRES driver fatfs sdmmc)

If you need help checking your build, let me know if you want me to write the code to initialize the 4-bit SDMMC parameters or help look for structural build warning errors within your VS Code compiler terminal log output.